Software Requirements Specification (SRS) for AI-Assisted Fiber Planning Client

1. Introduction

1.1 Purpose

This document defines the software requirements for the AI-Assisted Fiber Planning Client. The system supports both FTTH (Fiber-to-the-Home) split-based planning and non-FTTH point-to-point fiber network planning. It provides a web-based interface requiring only Python installation on the client side, with AI-assisted route planning performed by remote servers.

1.2 Scope

The system enables:

- AI-assisted fiber route planning (FTTH and non-FTTH).

- Manual map editing and SPOF analysis.

- Export of finalized plans in multiple formats (GeoJSON, KMZ, PDF, Image, Excel).

- Mandatory feedback submission to improve AI models and enable BOM/BOQ generation.

1.3 Definitions

- FTTH: Fiber-to-the-Home split-based architecture.

- Non-FTTH (P2P): Point-to-point fiber network without splitters.

- FDC: Fiber Distribution Cabinet.

- FDP: Fiber Distribution Point.

- ODF: Optical Distribution Frame.

- SPOF: Single Point of Failure.

2. Overall Description

2.1 Product Perspective

The client runs locally with Python and serves a browser-based UI. AI planning is performed by two remote servers:

- Server A: Always-on, low-spec, fast response for small jobs.

- Server B: On-demand, high-spec, used for large or complex jobs.

2.2 Product Functions

- Load and manage GeoJSON layers.

- Select sites for planning by clicking, entering coordinates, or selecting existing features.

- Edit and snap routes to OSM roads or existing infrastructure.

- Support FTTH options:

  - New FDC (split 1:4) with new FDP (split 1:8).

  - Existing FDC with new FDP.

  - New user ODF (split 1:32) at customer premise.

  - Tie DP (pull FDP without splitter from existing FDP or user ODF).

- Support non-FTTH point-to-point planning.

- SPOF tagging and calculation.

- Export finalized plans in GeoJSON, KMZ, PDF, Image, and Excel formats.

- Mandatory feedback submission before BOM/BOQ generation, with fallback if connectivity fails.

2.3 User Characteristics

Users are planners, engineers, or analysts familiar with fiber network design. They require intuitive map tools, reliable exports, and AI-assisted planning.

2.4 Constraints

- Client requires Python installation only.

- AI planning requires connectivity to remote servers.

- BOM/BOQ generation requires feedback submission.

3. Specific Requirements

3.1 Functional Requirements

- FR1: The client shall allow loading, viewing, and editing of GeoJSON layers.

- FR2: The client shall allow site selection by click, coordinate entry, or feature selection.

- FR3: The client shall support route editing with snapping to OSM roads or existing infrastructure.

- FR4: The client shall provide drawing modes for fiber deployment (existing pole, existing ductway, new pole, new ductway).

- FR5: The client shall support FTTH planning options (new/existing FDC, new ODF, tie DP).

- FR6: The client shall support non-FTTH point-to-point planning.

- FR7: The client shall allow SPOF tagging and calculate SPOF metrics.

- FR8: The client shall export finalized plans in GeoJSON, KMZ, PDF, Image, and Excel formats.

- FR9: The client shall enforce mandatory feedback submission before BOM/BOQ generation.

- FR10: The client shall allow BOM/BOQ generation locally if feedback submission fails due to connectivity.

3.2 Server Requirements

- SR1: Server A shall provide lightweight planning with <5s latency.

- SR2: Server B shall provide full planning for large jobs with <10 minutes latency.

- SR3: Both servers shall implement identical API contracts (/health, /v1/plan/sync, /v1/feedback).

- SR4: Servers shall include server_id and model_version in responses.

3.3 Non-Functional Requirements

- NFR1: The system shall use TLS for all communications.

- NFR2: The system shall use API keys/JWT for authentication.

- NFR3: The client shall minimize data sent to servers.

- NFR4: The client shall provide intuitive UI for server selection and planning mode selection.

- NFR5: The system shall cache OSM data locally to avoid rate limits.

4. Use Cases

Use Case 1: FTTH Planning with New FDC and FDP

- User loads GeoJSON layers.

- User selects site.

- User chooses FTTH option: New FDC (1:4) + new FDP (1:8).

- Client sends request to server.

- Server returns proposed routes.

- User edits and accepts plan.

- User submits feedback.

- Client generates BOM/BOQ and exports.

Use Case 2: Non-FTTH Point-to-Point Planning

- User loads GeoJSON layers.

- User selects two endpoints.

- Client sends request to server.

- Server returns direct route.

- User edits and accepts plan.

- User submits feedback.

- Client generates BOM/BOQ and exports.

Use Case 3: Offline Map Editing and Export

- User loads GeoJSON layers.

- User edits map manually.

- User decides no fiber planning required.

- Client allows export to GeoJSON, KMZ, PDF, Image, and Excel.

5. System Models

- Client-Server Architecture: Local client with Python + browser UI; remote AI servers for planning.

- Data Flow: Client → Server (planning request) → Server → Client (proposals) → Feedback submission → BOM/BOQ generation.

6. Risks and Mitigations

- Server B offline: Client shows status; user continues with Server A or manual planning.

- Connectivity failure during feedback: BOM/BOQ generated locally; feedback queued for retry.

- Data privacy: Explicit consent UI; minimize payloads.

- OSM rate limits: Cache road extracts locally.

- Feedback quality: Validate edits before retraining.

7. Next Steps

- Build prototype local server + frontend.

- Implement identical API endpoints on both servers.

- Develop lightweight planner for Server A.

- Deploy full planner model on Server B.

- Create UI for server selection and planning mode selection.

- Implement mandatory feedback submission before BOM/BOQ generation.

- Implement export routines for GeoJSON, KMZ, PDF, Image, and Excel.

- Pilot with sample datasets.

Conclusion: This SRS defines the requirements for a versatile AI-assisted fiber planning client supporting both FTTH and non-FTTH networks. It ensures offline usability, mandatory feedback submission, SPOF analysis, and secure, controlled use of remote AI servers.